# ##################   BONEMAT   #####################

## Elements

- main program: bonemat

- utilities:

- raw2SP

- raw2CT

- raw2RG

## Bonemat

To map dataset attributes over a finite element structured or unstructured mesh.

dataset

mesh

#### BONEMAT

parameters

frequency file

mesh + attributes

USAGE: bonemat param_file

param_file:

- input_mesh dataset output_mesh frequency_file // I/O files

- H0 r0 H1 r1	// HU Vs apparent density calibration points

- a b c		// Apparent density Vs Module of elasticity parameters

- Egap		// Difference of module between two materials

- steps		// number of integration steps for each direction

// inside an element

Bonemat package includes:

- Main program: bonemat.cxx;

- Modules: (*.h and *.cxx)

- mesh: to read, write and manage mesh informations.

- elements: to perform element computations.

- dataset: to read and manage dataset informations.

- hashtable: to manage node keys;

- Makefile: makefile

- Graphic interface: bonemat.tcl.

Mesh files can be represented as ASCII neutral format.

Mesh structure (see mesh.h):

- Nodes

- Key;

- Coordinates;

- Elements (see elements.h);

- Key;

- Nodes;

- Material;

- Materials;

- Key;

- Properties (density, Young’s module, etc.).

Supported element types:

- Tetrahedron;

- Hexahedron;

- Wedge;

- Quadratic tetrahedron (10 nodes);

tetrahedron

hexahedron

wedge

quadratic tetrahedron

Data sets are defined  in terms of structure and attributes (scalar data). See dataset.h.

Attributes: s0 s1 ... sxdim*ydim*zdim-1

Supported dataset types structures:

- SP (structured points)

- Dimensions: xdim ydim zdim

- Spacing: xspc yspc zspc

Origin: xorig yorig zorig

- CT (CT scan)

- Dimensions: xdim ydim zdim

- Spacing: xspc yspc

- Origin: xorig yorig

- Locations: z0 z1 ... zzdim-1

- RG (rectilinear grid)

- Dimensions: xdim ydim zdim

- X coordinates: x0 x1 ... xxdim-1

- Y coordinates: y0 y1 ... yydim-1

- Z coordinates: z0 z1 ... zzdim-1

structured points

CT scan

rectilinear grid

Dataset files can be represented as ASCII or binary "vtk like" format:

OLD:

Heading: # vtk dataFile version 2.0

Title: vtk output

Format: ASCII or BINARY

DATASET dataset_type

Structure

POINT_DATA num_point_data

SCALARS scalars_name scalars_type

Dummy line: LOOKUP_TABLE default

Scalar list

Heading: # vtk dataFile version 2.0

Title: vtk output

Format: ASCII or BINARY

See Type Below: 	DIMENSIONS …

SPACING …

ORIGIN …

POINT_DATA num_point_data

SCALARS scalars_name scalars_type

Dummy line: LOOKUP_TABLE default

Scalar list (actual data)

structured points

DIMENSIONS xdim ydim zdim

SPACING xspc yspc zspc

ORIGIN xorig yorig zorig

##### CT scan

DIMENSIONS xdim ydim zdim

SPACING xspc yspc

ORIGIN xorig yorig

LOCATIONS zlist

rectilinear grid

DIMENSIONS xdim ydim zdim

X_COORDINATES xlist

Y_COORDINATES ylist

Z_COORDINATES zlist

Graphic interface to manage param_files and to run application:

### Pseudocode

Mesh reading

Dataset reading

for each element in mesh

HU=HU(x,y,z)dVdV=HU(r,s,t)detJ(r,s,t)dV'V

where:

(r,s,t) is the local reference system centered at p0 node

(x,y,z)=wi(r,s,t)pi   for each node i

HU(x,y,z)=wi(x,y,z)HUi   for each point i of the dataset cell containing (x,y,z)

J(r,s,t)=∂x∂r∂x∂s∂x∂t∂y∂r∂y∂s∂y∂t∂z∂r∂z∂s∂y∂t

ρ=k1(H0,r0,H1,r1)HU+k0(H0,r0,H1,r1)

E=a+bρc

// computing materials (greedy strategy)

sorting elProp by E in decreasing order

matProp[0]= elProp[0]

E = E(matProp[0])

freq[0] = 0, i = 1

for each elProp j

if E – E(elProp[j]) > Egap

matProp[i]= elProp[j]

E = E(matProp[i])

freq[i] = 1

else

freq[i]++

mat(element(elProp[j])) = i

frequency file writing

mesh + attributes writing

## Utilities

To convert raw data set to "vtk like" format.

Raw2??

Raw dataset

Vtk like

dataset

USAGE:

raw2SP infile outfile xdim ydim zdim xspc yspc zspc xorig yorig zorig data_type BINARY/ASCII

raw2CT infile outfile xdim ydim zdim xspc yspc  xorig yorig locfile  data_type BINARY/ASCII

raw2RG infile outfile xdim ydim zdim xfile yfile zfile data_type BINARY/ASCII

data_type = unsigned_char, short or unsigned_short

### Graphic interfaces